Following topic will be used “devicereceive”

Make sure that  broker should have retain functionality, the message sent from server side to system, the message should be retained on broker and should be able to clear retained messages when ACK/NACK is sent

format    setTimedate:epoch|timezone    Reply:ACK/NACK

format   setFuelMeasurement:zerospan|fullspan  Reply:ACK/NACK

format   setSoCnDCur:SOC|CurrentOffset    Reply:ACK/NACK

format   setAutoManualMode:(0 - Auto   1 - Manual) Reply:ACK/NACK

format   setGenerator: (0 - GeneratorOff   1 - GeneratorOn)  Reply:ACK/NACK

format setFixSOCCharging:(0 - turnoff 1 - turnOm)

depending on tank type you can send different kind of format

All the values should be in meter for measurement

TankType = 0 (Cylnderical)

Tanktype = 1 (rectangular)

format setTankSettings:TankType|TotalLiters|Width|Length|Radius

Now if TankType = 0 you can only send Radius like this

setTankSettings:(TankType=0)|(TotalLiters=500)|(Radius=value in meter)

now if Tanktype=1 you can send width and length like following

setTankSettings:(TankType=1)|(TotalLiters=500)|(Width=value in meter)| (length=value in meter)

format   getSoCnDCur     Reply: SoC|CurrentOffset

format getSystemMode   Reply: Auto/Manual   0 - Auto   1 - Manual